船舶自動識別系統(A.I.S.)

張東明

前 言

有感於再度返回船隊服務，因職務上的機會，查覺到現職的船副們對於海事自動識別系統(A.I.S)設備的了解與掌握都有一定的落差，尤其對於系統內部的航次開航資料格式的輸入,總是會有如下的瑕疵:

1) 資料格式輸入錯誤: 未依照2004/12/15航行安全244章回所示範與要求的格式，正確的順序來輸入資料.

2) 輸入的資料不完整: 欠缺駛離之國名\港名\航往＞之箭頭符號，而只輸入目的地港名稱.

3) 港口名稱混淆不清: 自創港口名稱未使用國際統一的港口代碼或是欠缺輸入國家代碼(同一港口名稱卻是位於不同的地區如大陸天津的新港與臺灣的台中港均為港口統一代碼均為TXG,但是部份同仁自創台中港港口名為TCG).

4) 動態未能及時修正: 離港、抵港(時間)、下錨、起錨等動態，經常遺忘或未能及時修訂.

因此特別央請岸端資訊組同仁提供IMO PORT CODE與聯合國之各國地方代碼資料以及船副的幫忙資料輸入，然後親自再利用閒暇之餘統一彙整修訂，並參考目前船上所現有舊版的Guide To Port Entry 2005~2006來中文於國家代碼，前前後後歷經約4個月時間(其中尚欠缺部份資料無法完全翻註，這是心中有所遺憾與失落).

此舉乃希望在提供給其他海事同仁們，必需謹記在使用A.I.S.系統儀器輸入港口資料之際，應務必採納標準格式以及通用且一致協議認可的目的地代碼來輸入.以避開可能導致A.I.S.系統儀器程式因資料的混淆，造成其他航行中的船舶無法辨識; 或是於進港前受到港口管制中心的糾正警戒，或是靠泊港口期間，受檢於PSC而被列入缺點. 更甚者於航行中在臨近航路轉向點的叉路上,可能導致快速的追越船因誤判被追越船的動向而釀成大禍. 如高雄港水域、香港水域、中國大陸各港灣水域、新加坡新舊港口水域、馬六甲海峽口水域、可倫坡分道區口水域、蘇伊士運河口水域、西班牙直布羅陀口水域、美國洛山磯分道區口水域、加拿大溫哥華與美國華盛頓州的Juan De Fuca Strait的分道區水域、日本各港灣口水域、菲律濱馬尼拉南北槽分道區口.

備 註

最終(卸貨)港FINAL PORT: 大型船舶將貨物運載至卸貨之港.例如新加坡

目的地DESTINATION : 於新加坡卸下的貨物,再由其它較小的駁運船或交通工具轉運至其他港口或是收受方.

船舶自動識別系統(A.I.S.)

*全球各港口名稱的標準欄位格式的輸入

*聯合國編製的各國地方代縮碼UN LOCODE

- A.I.S.目的地訊息欄位如何輸入聯合國通用地點碼指南

GUIDANCE ON THE USE OF THE UN/LOCODE IN THE DESTINATION FIELD IN AIS MESSAGES

1). A.I.S.系統儀器程式因資料的輸入混淆不統一而影響辨識

每航程的開航之前,船副們可以在船舶自動識別系統儀器內,設定船舶未來的目的地,同時也可持續在系統儀器內做資料的更新. 根據統計證據資料顯示,船副們在A.I.S.系統儀器內輸入資料時,經常將同一個目的地的港口名,依認知分別使用不同的名稱來代表或是欠缺輸入國家碼(同一港口名稱卻是不同的國家).在這種情況下,將可能導致A.I.S.系統儀器程式因資料的混淆造成其他船舶無法辨識;進而影響目的地的無效交替變換,更甚者在航路轉向點的叉路上,可能導致快速追越船的誤判而釀成大禍.因此,當在輸入港口資料時,應務必採納標準格式以及通用且一致協議認可的目的地代碼來輸入.

The mariner is able to enter the ship’s destination into the A.I.S. at the start of each voyage, and to keep this information update. Evidence shows that mariners are using different names for the same location when entering destination date in their A.I.S. units. This situation leads to confusion and inefficiency in date interchange. Therefore, there is a need to harmonize date input when entering port information, by adopting an available universal protocol.

2). 聯合國通用地點碼之組成

此建議在現有的獨特目的地的欄位內,使用聯合國通用地點碼來輸入出發港與下一停靠港的代碼, 一般在目的地的資料欄位內, 共計可以輸入20個字元(母),而此處資料欄位的每一個字元,是使用6個位元(Bits)的美國標準資料交換碼所組成的.

It is recommended to use the existing destination field for entering both the port of departure and the next port of call(space for 20 characters of 6 bit ASCII is available), using the UN/LOCODE.

3). 各港口的標準輸入欄位資料格式

所推薦使用的聯合國通用地點碼格式,在前6個位置碼是代表出發國名及港名稱,包含一個區隔符號,緊接著是停靠下一港之國名及港名稱.

The recommended format is to indicate the port of departure at the first six positions of the date field followed by a separator and then the code for the next port of call

4). 區隔出發港與下一停靠港符號

為了辨識聯合國通用地點碼以及區別出發港與下一港的地點,兩港之間使用來區隔,如後之舉例: 此航次,某輪正航離中東的阿酋的杜拜港前往北歐的荷蘭鹿特丹港, A.I.S.系統儀器目的地資料欄位內使用聯合國通用地點碼:

In order to identify that it is a UN/LOCODE, to separate the locations and to indicate the ‘from’ and ‘to’ ports, a ‘>’ symbol should be used as a separator, see example below.

A ship is leaving Dubai bound for Rotterdam. Use of the UN/LOCODE would represent this voyage as below:

‘’AE DEX > NL RTM’

(1空隔)   (航往)  (1空隔)

AE DEX > NL RTM

(國名)  (港口名)  (國名)  (港口名)

5). 尚未確認下一停靠港口

假使不能確定下一停靠港的港口, 請在資料欄位輸入‘’?? ???’’,以替代聯合國通用地點碼,以為符合資料欄位的格式, 舉例如下列:

IF THE NEXT PORT OF CALL IS UNKNOWN, ‘’?? ???’’ should be entered instead of the UN / LOCODE in the corresponding place in the date field. See example below:

‘’AE DEX > ‘?? ???’

(1空隔)  (航往)  (1空隔)

AE DEX > ?? ???

(國名) (港口名) (未確定國名) (未確定港口名)

6). 聯合國通用地點碼尚未編寫的出發港港口之名稱

假使出發港港口之名稱,未編寫在聯合國通用地點碼內, 請在資料欄位輸入‘’XX XXX’’,以替代聯合國通用地點碼,以為符合資料欄位的格式, 舉例如下列:

If the port of departure does not have a designated UN/LOCODE, then ‘’XX XXX’’ should be entered instead of the UN/LOCODE. In the corresponding place in the date field. See example below:  ‘’XX XXX > ‘’US PBI’’

(1空隔)    (航往)  (1空隔)

XX XXX > US PBI

(無國名) (無港口名)  (國名) (港口名)

7). 聯合國通用地點碼尚未編寫出下一停靠港口之名稱\使用臨近港口名稱

假使下一停靠港港口之名稱,並未編寫入聯合國通用地點碼內時,請使用通用公認的英文明文之全名稱,輸入在目的地港港口的欄位內. 但在輸入目的地港港口的資料之前欄位,請先增加輸入‘’===’’(三個等長符號). 如連英文明文之全名都不能確認,請使用臨近的港口地點名稱,在此情形之下, 目的地港港口資料輸入欄位,也可能會排擠出發港港口的資料輸入欄位. 舉例如下列:

If the next port of call does not have a designated UN/LOCODE, the commonly accepted English name of the destination port should entered. Preceded by ‘’===’’ (‘’three equals signs’’). If no such name is known, the locally used name should be entered. In this case, there may not be enough space available to indicate the port of departure. See example below:

‘’===Orrvdken ’’

(1空隔)   (航往) (1空隔)

JP YOK > US ===ORRVDKEN

(國名) (港口名)  (國名)  (臨近的港口地點名稱)

8). 使用區域性名稱輸入格式

假使只能確認目的地名稱是通用的區域性全文名稱或縮代碼名稱,請在輸入使此資料之前的欄位,應先輸入三個等長符號‘’===’’ 舉例如下列:

If only the general area of destination is known the name or accepted abbreviation of the area preceded by ‘’===’’ (‘’three equals signs’’) should be entered. See example below:

‘’NL RTM >===US WC ’’

*Indicating a destination on the United States WEST Coast.

(1空隔)    (航往)       (1空隔)

NL RTM > === US WC

(荷蘭)  (鹿特丹)      (美國) (西海岸)

(三個等長符號)

二、日本版 - 適用於公海海上交通安全法之基準要求

Measures Subject to to Maritime Traffic Safety Act

1). 在船舶自動識別系統儀器內的設定目的地(港)資料欄位上,輸入代表最終卸貨港口(目的地)的縮代碼資料 :

Enter the symbol showing the destination port in the column for information on Destination (Final port) of A.I.S.

*適用對象與時機: 裝配有船舶自動識別系統的全球所有船舶,當船舶將準備航行於既訂航路時. 請在船舶自動識別系統儀器內的目的地(港)設定資料欄位上,輸入代表最終港的縮代碼資料

For those ships that will sailing on the route. enter the symbol showing the final port in the column for information on Destination

*例外對象: 當然不適用於未裝備船舶自動識別系統的船舶,或是被豁免勿需於24小時持續開機的船舶  Excluding those that are not equipped with A.I.S

2). 增加借道經過之航路(道)縮代碼資料

Add the symbol showing the via-route, if required

◎下錨泊碇: 依情況,如果船舶是要在的最終卸貨港內下錨 或是要在最終卸貨港港區附近的監控範圍內下錨.請增加輸入下錨的縮代碼資料

In case the ship is going to anchor in the port or in the vicinity of the boundary, add the symbol showing anchoring

(航往) (1空隔) (1空隔) > ○○ ○○○ ○○○○○ 1 2 國名 \ 最終卸貨港口名 取經航路 (1) 國 名: 二位縮碼 港 口 名 : 三位縮碼 (2) 取經航路(道): 三位縮碼 1). 國名\最終卸貨港口名Symbol showing the destination port 2). 取道經過之航路(道) Other symbol showing via-route, etc *最終卸貨港口名如符合適用港口規定的基準要求,則使用該基準要求,否則就使用聯合國標準港口地點碼 When the final port is a port to which the Act on port regulati ons applies, enter the symbol of the port as stipulated subject to the Act on port Regulations (if it is not, enter the UN LoCode) *如不清楚最終卸貨港口名稱時,請使用[?? ???],以替代聯合國標準港口地點碼 If the final port name is unknown enter ”?? ???” instead of the UN LoCode *如果也沒有聯合國標準港口地點碼,或是最終卸貨港口之名稱也不明確,則 可以在使用通稱,而在通稱之前加入=== (“===英文通稱”), 英文通稱也不明確時,請使用地區名稱 If the final port does not have the UN LoCode, or if the UN LoCode of the final port is unknown, enter the English name which is generally accepted for the final port concerned following the “===” mark. If such English name is unkown, enter the name which is used in that area

◎ 舉  例: 最終卸貨目的地為日本-橫濱港區第3區,靠JFE鋼鐵廠碼頭,進港之前先在東京灣-中之瀨海域-下錨泊碇

Example: Those ships whose final port is Keihin (Yokohama Section). Those ships that are going to anchor in the Nakanose sea area in Tokyo Bay. (In the port, the ship will sail heading for the pier of East Japan Works of JFE Steel Corporation in section 3)

(航往) (1空隔) 1空隔 區別符號 > JP YOK K/NNX 1 2 日本 \ 橫濱港 東京灣-中之瀨海域-下錨 1). 卸貨港為橫濱港區 The Final port is the Yokohama port 2). 進港前先在東京灣-中之瀨海域-下錨 The ship anchors in the sea area Nakanose in Tokyo Bay on its way. 船舶靠JFE鋼鐵廠碼頭,是適用日本港口規定的基準要求 Measures subject to the Act on Port Regulations in the destination port, the ship will be heading for the berthing facility of the JFWE pier

三、適用於日本國內之各港區之間港口規定的基準要求

Measures Subject to Act on Port Regulations

1). 在船舶自動識別系統儀器內的設定目的地(港)資料欄位上,輸入代表目的地(港)的縮代碼資料 :

Enter the symbol showing the destination port in the column for information on Destination of A.I.S.

*適用對象與時機: 裝配有船舶自動識別系統的全球所有船舶,當進入目的地港口內或在抵達目的地港港區附近的監控範圍內. 請在船舶自動識別系統儀器內的目地的港設定欄位上,輸入代表目的地(港)的縮代碼資料

For those ships sailing in the port concerned or in the vicinity of its boundary for the purpose of entering the port to which the Act on port regulations applies enter the symbol showing the destination port concerned in the column for information on Destination.

*例外對象: 當然不適用於未裝備船舶自動識別系統的船舶,或是被豁免勿需於24小時持續開機的船舶

Excluding those that are not equipped with A.I.S.

2). 請在進入特定目地港口內的航路(道)之前或在抵達特定目的地港港區附近的監控範圍的航路(道)上,除輸入航行線路的基礎上,應再增加輸入其附屬信號旗縮代碼資料

Add the symbol showing the route in the destination port or in the vicinity of the boundary thereof, if required.

*適用對象與時機: 如果船舶是要在特定的目的地港內下錨 或是要在目的地港港區附近的監控範圍內下錨.請一併額外的增加輸入下錨的縮代碼資料

In case the ship is going to anchor in the port or in the vicinity of the boundary, additionally enter the symbol showing anchoring.

*適用對象與時機: 如果船舶是要在較複雜的指定目的地港港灣內的狹窄航道上航行,請額外增加輸入航路(道)的縮代碼,以及信號代用旗的資料

In case the ship sails in some designated port, additionally enter the symbol showing the route in addition to the indication by a conventional code flag (substitute flag ) to indicate the route in the port.

3). 增加借道經過之航路(道)縮代碼資料

Add the symbol showing the via-route, if required.

*必要時當只是借道經過時請額外的增加輸入取道經過的航路(道)縮代碼資料

Further add and enter the via-route etc. If required.

4). 輸入目地港資料標準格式:

◎ 說 明: 在自動識別系統儀器內的目的地港資料欄位上,設定目的地港以及附屬資料,其格欄位式如下之順序解說

Entering Method: Column for information on destination of AIS.

(航往) (1空隔) (1空隔) (區別符號) > ○○ ○○○ ○○○○○ 1 2 3 國名 \ 港口名 航路(道) 取經航路(道) (1) 國 名: 二位縮碼 (1) 港口名: 三位縮碼 (2) 航路(道): 二位縮碼 (3) 經過航路(道): 三位縮碼 1). Symbol showing the destination port 2). Symbol showing the route in the port 3). Other symbol showing via-route, etc

◎ 舉  例: 目的地港為日本-博多港\停靠博多港第二區\途中取道經關門港之西出口與六連島之東,進入博多港

Example: Those ships whose destination port is Hakata port and, in Hataka port , heading for the berthing facility in section 2. They will be passing by the port, heading for the east of Mutsurejima at the West exit of Kanmon port.

(航往) (1空隔) 1空隔 區別符號 > JP HKT E2/WM 1 2 3 日本 \ 博多港 第二區 取經航路 1). The destination port is the Hakata 2). In the destination port, is is heading for the berthing facility in section 2 3). It passes by the Kanmon port in the westbound direction on its way

◎ 舉  例: 目的地港為日本-名古屋港(伊勢灣內Inner of the Ise wan),進港前,將在名古屋港港區附近的監控範圍內下錨泊碇

Example: Those ships whose destination port is Nagoya port. They will be in the vicinity of the boundary of the port before entering the port

(航往) (1空隔) 1空隔 > JP NGO OFF 1 2 日本 \ 名古屋港 港界附近下錨 1.: The destination port is the Nagoya port 2.: It will be anchoring in the In the vicinity of the boundary of the port before entering the port

翻編者: 張 明 東 2013/11/25 ~2014/03/12

參考資料: 1).航行安全244章回2004/DEC/15TH

2).日本AISへの入カコード表

3).聯合國各國地方代縮碼UN/LOCODE

4).2005~2006 Guide To Port Entry